Outsource Services required for Inspection & Repair of RAF-B2 Reservoir#5, RAF-A1 Reservoir#1&2

General Information:-

Qatar Electricity & Water Company (QEWC) is considered to be a forerunner among private sector companies involved in the field of Power Generation and Water Desalination in the Gulf region.

QEWC currently owns and operates following Plants:-

- RAF-B & B2 Power and Desalination Plants.

- RAF-A1 & A2 Desalination Plants.

Job Description

- In detailed check the reservoir in side wall and columns condition any crack exposed or structure damage surface

- To check the internal and external wall and floor expansion joints condition and sealant pored properly

- To check the reservoir ceiling condition and structural condition

- In detailed complete floor area check all expansion joint sealant pored / fixed and base floor structure.

- To check the wall & sealant around the inlet--outlet pipes embedded in the reservoir wall

- To check the condition of the Vertical & Horizontal sealant for the walls

- To check the condition of the lower portion of the concrete wall below cladding near kicker joints

- To check the roof top structural condition and parapet wall around the reservoir

- To check the outer wall paint condition and access stair case

- To  Check the paint of the roof top manhole covers and air vents etc

- Detailed Inspection report to be provided along with the Method of Statement for QEWC approval

- To carryout the repair works.

Terms & Conditions

- Supplier to visit QEWC site to assess the scope of work before submitting the offer

- Supplier should arrange the necessary Material required for the repair works & the Material should be Food Grade

- Supplier should arrange all the necessary Equipments, Tools & Tackles etc

- Contractor should provide Method of statement & Risk assessment before starting the job.

- Contractor will provide 5 years guarantee