Outsource Service to carry out SOW Repair/Replacing/Fixing of Corrugated Sandwich Sheets at RAF A1 SWPH ROOF

General Information:

Qatar Electricity & Water Company (QEWC) is considered to be a forerunner among private sector companies involved in the field of Power Generation and Water Desalination in the Gulf region.

QEWC currently owns and operates following plants at its Ras Abu Fontas Stations:

- RAF-B, B1 & B2 Power and Desalination Plants.

- RAF-A1,A2 & A3 Desalination Plants

Observation

- RAF A1 SWPH removal of existing roof sandwich panels and fixing new sandwich panels (material Double skin sandwich panel 0.4 mm PPGI top skin 50 mm PUR +0.35 mm PPGI bottom skin

| DC No. | Description |
| --- | --- |
| 400076 | "RAF-A1 SWPH Roof and Surrounding corrugated sheets following defects observed: 1).RAF-A1 SEA WATER PUMP HOUSE ROOF DAMAGED DURING HIGH WIND. 2)RAF-A1 SEA WATER PUMP HOUSE ROOF TOP SHED CORRUGATED SHEETs FOUND DISPLACED AND DAMAGED FROM MANY PLACES. 3)A1 SEA WATER PUMP HOUSE SHED SURROUNDING & NORTH SIDE ROOF SEVERAL PLACES CORRUGATED SHEETS ARE DAMAGED." . |

Supplier’s Scope of Work:

- Arrangement of accessible working plate form, menlifter or Scaffolding

- To remove the existing damage cladding sheets from the roof bottom

- Repair & surface preparation of the structure, application of Primer & Paint

- Supply & Installation of powder coated corrugated sandwich sheet

- Application of water proofing paint material on Joints & screws and overlap sheets

- To remove the unwanted material / debris site clear

TERMS AND CONDITIONS:

- Site visit to be carried out to evaluate the job before the furnishing the Offer.

- All the required resources like Skilled & Certified Manpower, Material, Manlifters, Scaffoldings, Tools & Tackles etc. need to be arranged by the Supplier

- The Material Supplied by the Supplier need to approve by QEWC before Installation

- Supplier should provide Method of Statement & Risk Assessment

- Supplier should follow QEWC safety regulation.

- The contractor staff should undergo induction training

- All the tools and equipment must have Portable Appliance Test